KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

COLLEGE OF ENGINEERING

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

GROUP 5

Dwira, Nhyira Ewuradwoa        7343023

Nkrumah Samuel                        7343623

Dontwi, Bismark Appiah            7342933

Koduah Hammond Paita            1872922

Nsagnan Richmond                     7343723

Alfred Amuzu                              1871722

Contents

3. PROBLEM STATEMENT	3

4. METHODOLOGY	4

5. DEMONSTRATION / SIMULATION	5

6. CONCLUSION	6

# 3. PROBLEM STATEMENT

Gas leakage remains a major safety concern, especially in residential and industrial environments. A notable example is the Apeate Explosion which occurred on 20th January 2022, resulting in 17 deaths and 59 severe injuries due to the absence of effective early warning systems. Similarly, a gas-related incident occurred at Oppong Hostel last year in one of the hostels at Obuasi campus, leading to injuries among students due to delayed detection and response.

These incidents triggered our interest for a real-time gas monitoring and alert system capable of detecting leaks early, notifying users, and triggering automatic safety actions to prevent loss of life and property.

# 4. METHODOLOGY

The system was designed using an ESP32 microcontroller integrated with an MQ gas sensor to continuously monitor LPG levels. The system processes sensor data and categorizes it into safe, warning, and critical levels, triggering appropriate responses such as alerts and automatic control actions.

Code & System Overview

Gas Sensor (GPIO32): Reads LPG levels

Calibration: Establishes baseline for accurate readings

Smoothing Algorithm: Improves accuracy using averaging

Threshold Logic:

< 40 ppm → SAFE

41–44 ppm → WARNING

≥ 45 ppm → CRITICAL

Output Control

LCD: Displays gas level and status

Green LED→ Safe

Yellow LED → Warning

Red LED → Danger

Purple LED→ Emergency / Lockdown

Buzzer: Audible alert

# 5. DEMONSTRATION / SIMULATION

- The system reads gas levels in real time

- Displays values on LCD and Blynk app

- Automatically changes system state based on thresholds

Output Behavior

System Response

< 40 ppm

Green LED, Safe display on LCD

41–44 ppm

Yellow LED, Warning on LCD, buzzer beep slightly

≥ 45 ppm

Red + Purple LED, buzzer ON, relay activated, fan ON

Key Takeaways

- Real-time detection improves safety

- Combination of analog processing and IoT enhances reliability

- Automatic response reduces human dependency

# 6. CONCLUSION

The project successfully demonstrates a reliable and cost-effective ESP32-based LPG gas detection system capable of real-time monitoring, alerting, and automatic safety response.

Key takeaways include the importance of early detection, integration of multiple alert systems, and the effectiveness of combining analog sensing with IoT-based monitoring.

However, challenges encountered included sensor calibration, fluctuating readings, and ensuring stable integration of multiple components.

In the future, improvements will focus on accurate PPM calibration, enhanced system stability, integration of advanced IoT features, and implementation of a fully automated gas shut-off mechanism for improved real-world deployment.